Instructions for 4Bank Conversion for FM91

TR 1

TR 2

Make sure it’s a 4518BP

I got mine from Dick Smith.

Connect “Clock Input” to Pin 10 of SK 403. (Economiser) and to Earth Via a 1k resistor.

Connect “Clock Enable” and “Vdd +5V”to Pin 2 of SK 408. (5v P)

Bridge output “Q3” with “Reset”

Connect “Vss 0V” to Ground (of course)

For a basic conversion:

Connect Output “Q1” to pin 26 of the 256k EPROM and to Earth Via a 1k resistor.

Connect Output “Q2” to pin 27 of the 256k EPROM and to Earth Via a 1k resistor.

You have to program economiser on the channels to activate the bank changing.

You have to put 4 different files in the EPROM.

Each with different Software Codes so the radio switches off and resets between each bank.

The start address for the 1st bank is 0000.

The start address for the 2nd bank is 2000.

The start address for the 3rd bank is 4000.

The start address for the 4th bank is 6000. Any problems, I can do one for you.

Get it

MORE ON NEXT PAGE……………..

……………….

A more complex way is to use some BC548’s and some LED’s to indicate which bank you are on.

Connect the collectors of the 2 BC548’s to Link 420. (5v Sw)

Instead of connecting the outputs Q1 and Q2 to the EPROM, connect them to the bases of TR 1 and TR 2.

The emitters of TR 1 and TR2 now go to the EPROM (Again using 1k resistors to earth) and to the blank volume control lines (via two 390 ohm resistors) that go to the head in the remote cable via SK 407 (you can find them, I think they’re “AF PA Input” and “RX AF Output). Make sure to cut the tracks on the SYNTH board to isolate them from the radio. Connect 2 LED’s on the pins in the remote head just behind the screen. They fit perfectly. One on RX AF I/P and the Other on RX AF O/P with the other sides to earth. The LED’s should indicate as follows;

NONE 	=	First Bank

LEFT 		=	Second Bank			Swap if these two

RIGHT	=	Third Bank			are mixed around.

BOTH 	=	Forth Bank

Mine does as all the others I’ve done.

I think that’s it. Good Luck….

AD